چکیده

یکی از روشهای مهم تعیین مقدار تنشهای برجا، آزمایش شکست هیدرولیکی است. این روش برای اولین بار به منظور افزایش تراوایی مخازن نفتی مورد استفاده قرار گرفت و پس از آن به منظور تخمین تنش برجا نیز استفاده شد. به منظور تحلیل پارامترهای حاصل از آزمایش، از روابط تحلیلی بایستی استفاده نمود، اما این روشها وجود ترک از پیش موجود و در نتیجه دو پارامتر مهم توزیع فشار درون ترک و زاویه آغاز رشد ترک را نمیتوانند در نظر گیرند. بنابراین مدل عددی به روش DDM به کمک نرم افزار MATLAB توسعه داده شد تا علاوه بر تخمین مقدار تنش افقی حداکثر، دو پارامتر مذکور را در محاسبات در نظر گیرد. علاوه بر این، آزمایشهای شکست هیدرولیکی در ساختگاه نیروگاه تلمبهذخیرهای آزاد-کردستان انجام و مقادیر تنشهای اصلی به کمک روابط تحلیلی در رویکرد کلاسیک مورد بررسی قرار گرفت. به طور کلی، مقادیر تنشهای افقی حداقل و قائم با افزایش عمق، روند افزایشی از خود نشان داد، اما مقادیر تنش افقی حداکثر از روند خاصی پیروی نکرد. همچنین مقدار تنشهای اصلی حاصل از رویکرد کلاسیک در ساختگاه تعیین شد (σh<σv≤σH). علاوه بر این، مقادیر تنش افقی حداکثر در مدل عددی تعیین و با مقادیر تنش حاصل از رویکرد کلاسیک مقایسه و اعتبارسنجی شد. در آخر، آنالیز حساسیت انجام شده بر روی تاثیر افزایش زاویه آغاز رشد ترک از 15-0 درجه در توزیعهای مختلف سیال درون ترک بر روی تنش افقی حداکثر نشان داد که افزایش زاویه آغاز ترک در این بازه، موجب افزایش تنش افقی حداکثر میشود.

Abstract

One of the most important methods for stress determination is hydraulic fracturing. This method was firstly introduced to increase permeability of petroleum reservoirs and then it was used for in-situ stress estimation. In order to analyze parameters of stress estimation test, analytical equations are often used however, such methods neglect pre-existing cracks and therefore the equations cannot consider two parameters including fluid pressure distribution along the cracks and inclination angle of cracks. Therefore, numerical modelling was developed in MATLAB software using DDM method to estimate magnitudes of maximum horizontal stresses considering the two later parameters. Besides, hydraulic fracturing tests were done in Azad pumped storage power plant of Kurdistan and magnitudes of principal stresses were determined using analytical method of classic approach. Overall, the magnitudes of minimum horizontal stresses and vertical stresses increased with the increase in depth however, the maximum horizontal principal stresses did not follow any special trend. It should be considered that the principal stress magnitudes of the site were determined using classic approach (σh<σv≤σH). Following this, the maximum horizontal principal stresses were determined using numerical code. It is then compared and validated with classical approach stress magnitudes. Finally, sensitivity analysis was done on the effect of increase in crack inclination angle from 0-15° with different fluid pressure distributions along cracks and maximum horizontal stress. It is concluded that the increase in crack inclination angle within the period lead to the increase in maximum horizontal stress magnitude.